Practical - 02

Title: - Program to simulate traffic between two nodes

Aim: - To simulate traffic between two nodes

Lab Objectives: -

To get familiarize with a Network Simulation Tool

Description: -

A network is two or more devices connected through links. A link is a communications pathway that transfers data from one device to another.

There are two possible types of connections:

- point-to-point

- multipoint.

Point-to-point connection

Point-to-point connection provides a dedicated link between two devices.

The entire capacity of the link is reserved for transmission between those two devices.

Use an actual length of wire or cable to connect the two ends, but other options, such as microwave or satellite links, are also possible.

For example, When you change television channels by infrared remote control, you are establishing a point-to-point connection between the remote control and the television's control system.

Multipoint

A multipoint (also called multidrop) connection is one in which more than two specific devices share a single link

In a multipoint environment, the capacity of the channel is shared, either spatially or temporally.

If several devices can use the link simultaneously, it is a spatially shared connection.

If users must take turns, it is a timeshared connection.

Exercises

1. Write a program to implement the given point to point topology and simulate traffic between two nodes.

10.0.0.1

10.0.0.2

Code:

/*-*- Mode:C++; c-file-style:"gnu"; indent-tabs-mode:nil; -*-*/

// add required header files

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

#include "ns3/csma-module.h"

#include "ns3/ipv4-global-routing-helper.h"

// Adding namespace declaration

using namespace ns3;

//Define log component where log msgs will be saved

NS_LOG_COMPONENT_DEFINE("p2pExample");

// Main function

int main(int argc, char *argv[]){

// read the command line arguments

CommandLine cmd(__FILE__);

// Process the command line arguments

cmd.Parse(argc, argv);

// Set time Resolution to 1 nano second

Time::SetResolution(Time::NS);

// Logging

LogComponentEnable("UdpEchoClientApplication",LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication",LOG_LEVEL_INFO);

// Create NodeContainer object to store our nodes

NodeContainer nodes;

// Create 2 nodes

nodes.Create(2);

// create object of the point-to-point helper object class to configure net device and the channels

PointToPointHelper pointToPoint;

// Configure the net Device

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps")); // Set Data Rate

// Configure the Channel

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms")); // Set Delay Attribute

// Install net devices on nodes

NetDeviceContainer devices;

devices=pointToPoint.Install(nodes);

// install netdevices on node and connect with the Channels

// Configure and Install protocol suits on nodes

InternetStackHelper stack;

stack.Install (nodes);

// configure network IP address and subnet mask for network

Ipv4AddressHelper address;

// set data

address.SetBase("10.0.0.0","255.0.0.0");

// Assign IP addresses to the interfaces of netDevices

Ipv4InterfaceContainer interfaces = address.Assign(devices);

// Configure our Applications

// Configure UDPEchoServerApplication

UdpEchoServerHelper echoServer(9); // Setting port number of server application

// Application Container create object to store server application and install on node(1)

ApplicationContainer serverApp = echoServer.Install(nodes.Get(1)); // indexed 1 server

// Configure start and stop time of server Application

serverApp.Start(Seconds(1.0)); // server app should start first

serverApp.Stop(Seconds(10.0)); // server app should stop

// Configure UdpEchoClientApplication

UdpEchoClientHelper echoClient(interfaces.GetAddress(1),9);

// Configure the attribute of client Application

echoClient.SetAttribute("MaxPackets", UintegerValue (1));

echoClient.SetAttribute("Interval", TimeValue (Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue (1024));

// Install Client Application on Node 0

ApplicationContainer clientApp = echoClient.Install(nodes.Get(0));

// Enables Routing between two networks 10.0.0.0 and 20.0.0.0

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

// for Running the code

AnimationInterface anim("p2pAniExcer.xml");

anim.SetConstantPosition(nodes.Get(0),20.0,30.0);

anim.SetConstantPosition(nodes.Get(1),40.0,30.0);

// Configure Start and Stop Time

clientApp.Start(Seconds(2.0));

clientApp.Stop(Seconds(10.0));

// Simulation on Run and start

Simulator::Run();

// Destory this Resourses

Simulator::Destroy();

return 0;

}

Output

Conclusion: Implemented point to point topology and simulate traffic between two nodes

After performing this Practical/lab, students are expected to answer the following questions.

- What is the Boilerplate?

- What are the seven levels of log messages?

- Explain the following classes and methods in those classes which are used in your script.

  - NodeContainer

  - PointToPointHelper

  - NetDeviceContainer

  - InternetStackHelper

  - Ipv4AddressHelper

  - Ipv4InterfaceContainer

Reference

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_node_container.html#details

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_point_to_point_helper.html#details

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_net_device_container.html

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_internet_stack_helper.html

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_ipv4_address_helper.html

https://www.nsnam.org/docs/release/3.15/doxygen/classns3_1_1_ipv4_interface_container.html

https://www.nsnam.org/docs/tutorial/html/conceptual-overview.html

https://www.nsnam.org/docs/release/3.7/tutorial/tutorial_21.html